SGI030/035

Sensor head

Specifications

| Model |  | SGI030/035 |
| --- | --- | --- |
| Reference distance (RF) |  | 30mm |
| Measuring range |  | -5mm-5mm |
| Light source | Light source wavelength | 655nm |
|  | Laser class | Class II |
|  | Laser output power | 5mW |
| Laser point diameter (Light dot size) | Small laser point | Φ60μm |
|  | Wide laser point | 60*400μm |
| Repeatability① |  | 0.05um |
| Linearity |  | ±0.05% F.S. |
| Temperature characteristics |  | 0.01% F.S./℃ |
| Sampling refresh rate (Hz) |  | 1/2/5/10/20/50/88kHz (Choose from 7 levels) |
| I/O | Communication port | 1 100Base-TX/1000Base-T Ethernet interface, 1 RS485 |
|  | Analog output | 1-channel analog outputs, supports analog voltage and analog current switching |
|  | IO input | 2 channels, timing, zeroing and single-ended encoder function multiplexing. |
|  | IO output | 4 channel, determines status of output. |
| Operating temperature |  | 0-50oC |
| Storage temperature |  | -20-70oC |
| Working humidity |  | 35%-85% no condensation |
| ESD protection |  | Contact discharge 4kV, air discharge 8kV, conforms with IEC61000-4-2 standard |
| EFT protection |  | Power port 2kV/5 or 100kHz, signal port 1kV/5 or 100kHz, conforms with IEC61000-4-4 |
| Bumps |  | 50Gs/3ms on each axis, conforms with IEC 68-2-27 Ea |
| Vibrations |  | 10Gs (10-500Hz), conforms with IEC 68-2-6 Fc |
| IP level |  | IP67, conforms with IEC 60529 standard |
| Dimensions (mm) |  | 90x75x41 |
| Data cable (wiring) model |  | SCB-GICAM-HA1-3m/6m/10m |
| Weight (Including cables) |  | 324g |

Note: ① Obtained from 4096 tests at reference distance.

Product Dimensions

2-Ø4.40  through mounting hole position